Applications

PROSE optical fiber raceway is a set of products that can provide full protection for optical fibers and facilitate operation. The basic components include: channel body, connectors, horizontal elbows, vertical elbows, horizontal tees, horizontal crosses, vertical tees, adapters, terminal end caps, fiber outlets, bell mouths, and various support components. Featuring a standardized modular design with high engineering precision, it offers rapid, flexible, and comprehensive protection for optical fibers.

Characteristic

- Innovative design, full arc dead angle protection fiber optic cable.

- Tool-free installation, barrier-free connection.

- Multiple fiber outlet specifications to meet fiber requirements in various locations.

- Powerful compatibility, worry-free expansion.

- UL94-V0 highest-level flame-retardant material.

Straight.Duct

Straight Duct

| Product Number | CR24-SD-111-11(Body) CR24-SD-110-11(Cover) |
| --- | --- |
| Size | 240mm (W) x 100mm (H) x 2000(mm) (L) |
| Flame retardant level | UL94-V0 |
| External dimensions | Body: 1278×118×2000(mm), Cover: 274×12×2000(mm) |
| Maximum payout | 5800 strands (3 mm wire) |

Duct Connector

Duct Connector

| Product Number | CR24-CO-211-11 |
| --- | --- |
| Specification dimensions | 240×100(mm) |
| Flame retardant level | UL94-V0 |
| Scope of application | Connection of components of 240-width Straight Duct |
| External dimensions | 287×60×119(mm) |
| Installation requirements | PR-LT5B4 Plastic round handle bolt ×8 |

Horizontal Corner Bend

Horizontal 90° Corner Bend

| Product Number | CR24-CB-311-11(Body), CR24-CB-310-11 (Cover) |
| --- | --- |
| Specification dimensions | 240×100(mm) |
| Flame retardant level | UL94-V0 |
| Scope of application | Applicable to 240 series Straight Duct horizontal turns |
| External dimensions | Body: 347×347×121(mm), Cover: 381×381×37(mm) |
| Installation requirements | 2 sets of fittings; cover plate optional |

Horizontal Tee

Horizontal Tee

| Product Number | CR24-TE-411-11(Body), CR24-TE-410-11 (Cover) |
| --- | --- |
| Specification dimensions | 240×100(mm) |
| Flame retardant level | UL94-V0 |
| Scope of application | Applicable to 240 series Straight Duct horizontal T-shaped connection |
| External dimensions | Body: 410×347×121(mm), Cover: 480×381×37(mm) |
| Installation requirements | 3 sets of fittings; cover optional |

Horizontal Cross

Horizontal Cross

| Product Number | CR24-CR-511-11(Body), CR24-CR-510-11 (Cover) |
| --- | --- |
| Specification dimensions | 240×100(mm) |
| Flame retardant level | UL94-V0 |
| Scope of application | Applicable to 240 series Straight Duct horizontal Cross |
| External dimensions | Body: 410×410×121(mm), Cover: 480×480×37 (mm) |
| Installation requirements | 4 sets of fittings; cover optional |

End Closure

End Closure

| Product Number | CR24-EC-611-11 |
| --- | --- |
| Specification dimensions | 240×100(mm) |
| Flame retardant level | UL94-V0 |
| Scope of application | Applicable to 240 series Straight Duct End Closure |
| External dimensions | 270×25×116(mm) |
| Installation requirements | - PR-LT5B4 Plastic round handle bolt ×4 - PR-LT338 Plastic square nut ×4 |

45° Upward Elbow

| Product Number | CR24-AE-710-11 (Cover) |
| --- | --- |
| Specification dimensions | 240×100(mm) |
| Flame retardant level | UL94-V0 |
| Scope of application | Applicable to 240 series Straight Duct 45° Upward Elbow |
| External dimensions | Body: 320×287×121(mm), Cover: 290×325×40(mm) |
| Installation requirements | 2 sets of fittings; cover optional |

45° Upward Elbow

45° Upward Elbow

| Product Number | CR24-AE-711-11(Body) |
| --- | --- |
| Specification dimensions | 240×100(mm) |
| Flame retardant level | UL94-V0 |
| Scope of application | Applicable to 240 series Straight Duct 45° Upward Elbow |
| External dimensions | Body: 320×287×121(mm), Cover: 290×325×40(mm) |
| Installation requirements | 2 sets of fittings; cover optional |

| Product Number | CR24-AE-720-11 (Cover) |
| --- | --- |
| Specification dimensions | 240×100(mm) |
| Flame retardant level | UL94-V0 |
| Scope of application | Applicable to 240 series Straight Duct 45° Downward Elbow |
| External dimensions | Body: 277×187×162(mm), Cover: 286×199×100(mm) |
| Installation requirements | 2 sets of fittings; cover optional |

45° Downward Elbow

45° Downward Elbow

| Product Number | CR24-AE-721-11(Body) |
| --- | --- |
| Specification dimensions | 240×100(mm) |
| Flame retardant level | UL94-V0 |
| Scope of application | Applicable to 240 series Straight Duct 45° Downward Elbow |
| External dimensions | Body: 277×187×162(mm), Cover: 286×199×100(mm) |
| Installation requirements | 2 sets of fittings; cover optional |

Waterfall Outlet

| Series | 90*170mm Series | 120*170mm Series | 130*170mm Series |
| --- | --- | --- | --- |
| Product Number(Body) | CRAC-WO-851-11 | CRAC-WO-861-11 | CRAC-WO-871-11 |
| Product Number(Cover) | CRAC-WO-850-11 | CRAC-WO-860-11 | CRAC-WO-870-11 |
| External dimensions(Body) | 186×170×130mm | 250×211×146mm | 226×170×130mm |
| External dimensions(Cover) | 108×103×153mm | 258×260×149mm | 148×103×153mm |
| Flame retardant level | UL94-V0 | UL94-V0 | UL94-V0 |
| Color | Yellow | Yellow | Yellow |
| Materials | PVC / ABS | PVC / ABS | PVC / ABS |
| Corrugated Tube Size | 55mm | 55mm/80mm | 55mm/80mm |
| Installation requirements | Round Plastic Screw with Nut ×2set |  |  |

Corrugated Tube Kit

| Product Number | CRACT-911-11(Φ55mm), CRACT-921-11(Φ80mm) |
| --- | --- |
| Color | Yellow |
| Materials | PVC / ABS |
| Installation requirements | Position fixer×1set |

Screw and Nut

Round Plastic Screw and Nut

| Product Number | CRAC-SN-211-11/CRAC-SN-221-11 CRAC-SN-221-11 CRAC-SN-221-11 CRAC-SN-221-11 CRAC-SN-221-11 CRAC-SN-221-11 CRAC-SN-221-11 CRAC-SN-221-11 CRAC-SN-221-11 / |
| --- | --- |
| Specification dimensions(mm) | M5×12(Screw); M5(Nut) |
| External dimensions(mm) | Φ25×24mm (Screw); 19×13x2.5mm (Nut) |

Supporting Assembly

| Product Number | CRAC-SA-XXX-XX |
| --- | --- |
| Accessories | - T-shaped Plastic Screw ×2 - Plastic knob Nut × 2 - “U” types of overhead cranes × 2 - Plastic double-hole crossarm × 1 - Expansion screws × 4 - Threaded rod × 2 - Flange nuts × 8 |

GLOBAL FOOTPRINT

HQ

PRODUCTION / OPERATIONAL HUB

R&D / ENGINEERING

SUBSIDIARIES / SALES REPRESENTATIVES

PROSE Technologies

www.ProseTechnologies.com

info@ProseTechnologies.com

© 2026 PROSE Technologies

04/26

For more information

refer to our website