Description

PROSE Fiber Optic Patch Cords provide high-performance optical connectivity for data centers, telecommunications, and enterprise networks.  Featuring low insertion loss, high return loss, and precision-polished connectors, they ensure reliable and efficient signal transmission across a wide range of fiber optic applications.

Key Features

- PVC, LSZH and other outer sheath materials can be selected.

- OFNR, OFNP and other flame retardant grades can be selected.

- Low insertion loss, high return loss.

- Two types of end grinding mode: UPC, APC.

- Available with simplex cable, duplex cable, mini-zip cord cable, ribbon cable, other types of cables on request

- Separately packed in plastic bag, including serial number and attenuation data.

- Connector types: LC, SC, FC, ST, E200, MPO.

Profile/Outline

E2000-E2000

SC-SC

LC-LC

E2000-SC

E2000-LC

MPO-MPO

Technical Parameters

| Fiber types | Single-mode 9/125um (G652D,G657A/B, etc.) Multi-mode 50/125um (OM3/OM4/OM5) |
| --- | --- |
| Connecter variants | Simplex and duplex |
| External diameter | 0.9mm buffered fiber 2.0mm simplex/duplex cable 3.0mm simplex/duplex cable |
| Delivery form | Single item in foil bag One product ID label with measurement values per patch cord Serial number label on patch cord |
| Ferrule Outside Diameter (mm) | 1.25mm (LC); 2.50mm (SC) |

Reference Standards

| LC | Comply with IEC 61754-20 |
| --- | --- |
| SC | Comply with TIA 568, IEC 874-10 |
| FC | Comply with IEC 60874-7 |
| ST | Comply with TIA 568, IEC 874-14 |
| The insertion loss test | Acc.to IEC 61300-3-4 |
| The return loss test | Acc.to IEC 61300-3-6 |

Optical cable parameters

| Type |  | Insertion loss（dB） | Return Loss（dB） |
| --- | --- | --- | --- |
| SM:SC; FC; ST; LC,E2000 | Ultra (UPC) | ＜0.30 dB | ＞50 dB |
|  | Angle(APC) | ＜0.30 dB | ＞60 dB |
| MM:SC; FC; ST; LC,E2000 | Ultra (UPC) | ＜0.30 dB | ＞25 dB |

| Type | Parameters |  |
| --- | --- | --- |
| Core | 1 | 2 |
| Tensile strength (N) | 150 | 300 |
| Compressive strength (N/cm) | 15 | 30 |
| Minimum Bending Radius for Installation(mm) | 20 times O.D | 20 times O.D |
| Minimum Bending Radius for Operation (mm) | 10 times O.D | 10 times O.D |
| Installation Temperature(℃) | 0-50℃ | 0-50℃ |
| Operating Temperature(℃) | -20 to +60℃ | -20 to +60℃ |
| Transportation and Storage Temperature(℃) | -25 to +70℃ | -25 to +70℃ |

Ordering Information

| Patch Cord &Pigtail | Fiber Type | - | Fiber Count | Connector L | Connector R | Cable Structure | Outer Sheath | - | Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FJ: Fiber Patchcord | GD:G652D A1:G657A1 A2:G657A2 B2:G657B2 B3:G657B3 M2:OM2 M3:OM3 M4:OM4 M5:OM5 |  | 01:1F 02:2F 04:4F 06:6F 08:8F 12:12F | LC:LC UPC SC:SC UPC FC:FC UPC ST:ST UPC LA:LC/APC SA:SC/APC FA:FC/APC MM:MPO Male MF:MPO Female E2:E2000/APC | 00:For Pigtail LC:LC UPC SC:SC UPC FC:FC UPC ST:ST UPC LA:LC/APC SA:SC/APC FA:FC/APC MM:MPO Male MF:MPO Female E2:E2000/APC | 1:0.9mm tight-buffered cable 2:2.0mm 3:3.0mm 4:2*2.0mm | 1:OFN 2:LSZH 3:OFNR 4:OFNP |  | 0005:0.5m 0010:1m 0050:5m 0100:10m 0150:15m 0200:20m |

For Example:

| Order Number | Description |
| --- | --- |
| Singlemode |  |
| FJGD-02LCLC42-0020 | Patchcord, Duplex, LC-LC, SM G652D, 2*2.0mm, LSZH, 2m |
| FJGD-01SCSC22-0020 | Patchcord, Simplex, SC-SC, SM G652D, 2.0mm, LSZH, 2m |
| FJGD-01LCSC22-0020 | Patchcord, Simplex, LC-SC, SM G652D, 2.0mm, LSZH, 2m |
| FJGD-01E2E222-0020 | Patchcord, Simplex, E2000-E2000, SM G652D, 2.0mm, LSZH, 2m |
| FJGD-01LCE222-0020 | Patchcord, Simplex, LC-E2000, SM G652D, 2.0mm, LSZH, 2m |
| FJGD-01SCE222-0020 | Patchcord, Simplex, SC-E2000, SM G652D, 2.0mm, LSZH, 2m |
| Multimode |  |
| FJM3-02LCLC42-0020 | Patchcord, Duplex, LC-LC, MM OM3, 2*2.0mm, LSZH, 2m |
| FJM3-01SCSC22-0020 | Patchcord, Simplex, SC-SC, MM OM3, 2.0mm, LSZH, 2m |
| FJM3-01LCSC22-0020 | Patchcord, Simplex, LC-SC, MM OM3, 2.0mm, LSZH, 2m |
| FJM3-01LCLC22-0020 | Patchcord, Simplex, E2000-E2000, MM OM3, 2.0mm, LSZH, 2m |
| FJM3-01LCE222-0020 | Patchcord, Simplex, LC-E2000, MM OM3, 2.0mm, LSZH, 2m |
| FJM3-01SCE222-0020 | Patchcord, Simplex, SC-E2000, MM OM3, 2.0mm, LSZH, 2m |

GLOBAL FOOTPRINT

HQ

PRODUCTION / OPERATIONAL HUB

R&D / ENGINEERING

SUBSIDIARIES / SALES REPRESENTATIVES

PROSE Technologies

www.ProseTechnologies.com

info@ProseTechnologies.com

© 2026 PROSE Technologies

04/26

For more information

refer to our website